# Akvo Green – Manual: setting up a new site

Oct 13, 2026 · @Christian

## Overview

A new site takes about 10 steps and ends with one Raspberry Pi publishing to its own AWS stack and its own web dashboard. Sites share the AWS account and the code (repos Akvo_Green and VenkoDemo), never data, buckets, API keys or URLs.

Everything is named after one site key: short, lowercase, typed the same way everywhere. The examples in this manual set up a site called plant5; the reference site is cmd4.

| Item | Rule | Example (plant5) |
| --- | --- | --- |
| Site key | lowercase, no spaces | plant5 |
| Gateway id = MQTT client id | AKVO_<key>, unique per Pi | AKVO_plant5 |
| MQTT topics | VENKO/<key>/data, VENKO/<key>/system | VENKO/plant5/data |
| Gateway config folder | Akvo_Green/sites/<key>/config_data/ | sites/plant5/config_data/ |
| Certificates (git-ignored) | Akvo_Green/sites/<key>/certs/ | sites/plant5/certs/ |
| IoT policy / thing | venko-<key>-gateway / AKVO_<key> | venko-plant5-gateway |
| AWS stack / samconfig env | venko-<key> / <key> | venko-plant5 / plant5 |
| Data bucket | venko-<key>-<account>-<region> | venko-plant5-884520769610-us-east-1 |
| Web bucket | venko-<key>-web-<account> | venko-plant5-web-884520769610 |
| Athena / Glue database | venko_<key> | venko_plant5 |
| Dashboard labels | VenkoDemo/config/sites/<key>.json | config/sites/plant5.json |

Case matters: folder names, topics, IoT policy resources and bucket names are all case-sensitive. A folder sites/Plant5 with topics VENKO/plant5/* doesn't fail loudly; the Pi is just not allowed to publish.

setup flow · 9 steps across the Pi, your computer and AWS

You work mostly from your computer: the Pi is touched at the start (SD card) and the end (install), AWS in the middle.

Set these shell variables once on your computer; every command below uses them:

KEY=plant5; CLIENT=AKVO_plant5; ACCT=884520769610

## What you need

Hardware at the site

| Part | Notes |
| --- | --- |
| Raspberry Pi 4, 3B+ or CM5 + a 16 GB+ SD card | Pi 4 if it will drive a monitor |
| Pi power supply, 5 V 2.4 A or more | Short, thick cable; the official 5.1 V 3 A gives the most margin |
| RS-485 interface | USB-RS485 adapter (/dev/ttyUSB0) or Waveshare 2-CH isolated RS485 HAT (/dev/ttySC0) |
| Sensor power supply, 12 V | Separate from the Pi. For a bus like cmd4's (7 devices, one with a UV lamp and wiper) use 2.5–3 A |
| Optional: 1.54" I2C OLED (SSD1309), monitor | The monitor needs its own 5 V 3 A supply, never a Pi USB port |

On your computer

- Both repos side by side: ~/git_reps/claude/projects/Venko/Akvo_Green and .../Venko/VenkoDemo.

- AWS CLI logged in to account 884520769610, region us-east-1 (aws sts get-caller-identity must work).

- AWS SAM CLI and Python 3 (for sam build/sam deploy and the config tools).

- An SSH key (~/.ssh/id_*.pub) to paste into Raspberry Pi Imager.

- Raspberry Pi Imager.

## Step 1 – Write the SD card

In Raspberry Pi Imager, on any computer:

- OS: Raspberry Pi OS (64-bit) with desktop. The desktop is needed for the AKVO Modbus Tool and the dashboard kiosk; Lite works if you need neither.

- Edit settings:

  - hostname plant5;

  - user pi and a password;

  - WiFi SSID, password and country (skip on Ethernet);

  - time zone America/Mexico_City;

  - Services: enable SSH, public-key only, paste your ~/.ssh/id_*.pub.

- Write the card, boot the Pi, find its address (ping plant5.local or the router's client list), and reserve that IP on the router.

## Step 2 – Prepare the Pi

ssh pi@192.168.68.140            # your Pi's address
sudo apt update && sudo apt full-upgrade -y && sudo reboot

After the reboot, check the RS-485 interface:

- USB adapter: ls /dev/ttyUSB* shows /dev/ttyUSB0.

- Waveshare 2-CH HAT: set its DIP switch to auto, add this line under [all] in /boot/firmware/config.txt, reboot, then ls /dev/ttySC* shows ttySC0 and ttySC1:

dtoverlay=sc16is75x-spi,sc16is752,spi1-1cs,spi1-0,int_pin=24

- OLED (optional): also add dtparam=i2c_arm=on, reboot, and i2cdetect -y 1 shows 3c.

## Step 3 – Gateway config (Akvo_Green)

Start from cmd4's four CSV files, edit them, then build config.json:

cd ~/git_reps/claude/projects/Venko/Akvo_Green
mkdir -p sites/$KEY/config_data
cp sites/cmd4/config_data/{devices,modbus,system,aws}.csv sites/$KEY/config_data/

The files must be inside sites/$KEY/config_data/, not sites/$KEY/.

system.csv – gateway identity, timing and optional features:

gateway_id,city,poll_interval,system_interval,watchdog_timeout,database_enabled,database_retention_days,web_enabled,web_port,wifi1_ssid,wifi2_ssid
AKVO_plant5,America/Mexico_City,20,60,120,1,30,1,8080,Planta,Oficina

- gateway_id = $CLIENT. poll_interval = seconds between sensor reads.

- database_enabled=1 keeps a local history on the Pi; web_enabled=1 serves it at http://<pi>:8080.

- wifi1_ssid / wifi2_ssid = WiFi networks to join, preferred first (step 4). Leave the columns out for none.

- Optional OLED: add display_enabled,display_driver,display_address = 1,ssd1309,0x3C.

modbus.csv – the RS-485 bus:

port,baudrate,timeout,parity,stopbits,bytesize,simulate_enabled
/dev/ttyUSB0,9600,1.0,N,1,8,1

- port: /dev/ttyUSB0 (USB adapter) or /dev/ttySC0 (HAT channel 1).

- simulate_enabled: 1 = synthetic demo data, 0 = real sensors. See the real-sensor known issue under Common problems before using 0.

devices.csv – one row per sensor; rows with the same device_id form one device:

device_id,slave,sensor_name,addr,count,scale,offset,unit,type,min,max,enabled
DEV_1,2,Pt100Sensor,0,1,1,0,°C,float,0,55,1
DEV_3,3,Humedad,1,1,1,0,%HR,float,0,80,1
DEV_5,25,PhSensor,0,2,1,0,unit,float,0,14,1

- slave = Modbus address; addr / count = first holding register and how many.

- type: float, float32, int, uint16, uint32, int32; only the float types apply scale and offset.

- min / max = alarm limits (also the OLED's ▲/▼). enabled=0 skips the row.

- Several sensors on one physical unit share a slave; the build then warns Duplicate slave ID, which is harmless.

aws.csv – where to publish:

host,client_id,ca,cert,key,topic_pub,topic_system,dashboard_url,data_bucket,web_bucket
a2zzu55sjawy4x-ats.iot.us-east-1.amazonaws.com,AKVO_plant5,./certs/AmazonRootCA1.pem,./certs/certificate.pem.crt,./certs/private.pem.key,VENKO/plant5/data,VENKO/plant5/system,,venko-plant5-884520769610-us-east-1,venko-plant5-web-884520769610

- host is the same for every site; keep the three cert paths as they are.

- Leave dashboard_url empty until step 6 gives it.

Build and validate:

cd gateway && python3 config_manager.py build --config-dir ../sites/$KEY/config_data && cd ..

Expected: Loaded N devices and Validation successful.

## Step 4 – WiFi networks (optional)

The Pi can know two WiFi networks, preferred first; you change them from your computer instead of re-imaging the SD card. Only the SSIDs go in system.csv (step 3). The passwords go in sites/$KEY/wifi.csv, which is git-ignored like the certificates and never reaches config.json:

ssid,password
Planta,your-planta-password
Oficina,your-oficina-password

chmod 600 sites/$KEY/wifi.csv

- One row per SSID in system.csv; an empty password means an open network.

- WPA passwords are 8–63 characters. A placeholder like CHANGE_ME is refused on push.

- On the next push (step 8) they become NetworkManager profiles akvo-wifi1 (priority 20) and akvo-wifi2 (priority 10). The current connection isn't dropped; the Pi switches when its network goes away or at the next reboot.

- To change WiFi later: edit system.csv and wifi.csv, rebuild, push.

## Step 5 – IoT certificate, policy and thing (AWS)

Each Pi gets its own certificate and a policy that only lets it connect as $CLIENT and publish to VENKO/$KEY/*. The private key can only be downloaded now; it goes in sites/$KEY/certs/ (git-ignored) and on the Pi, nowhere else. Run from the Akvo_Green root:

mkdir -p sites/$KEY/certs && cd sites/$KEY/certs && umask 077
aws iot create-keys-and-certificate --set-as-active \
  --certificate-pem-outfile certificate.pem.crt --private-key-outfile private.pem.key \
  --query certificateArn --output text > cert.arn
cp ../../cmd4/certs/AmazonRootCA1.pem .
cat > policy.json <<JSON
{ "Version": "2012-10-17", "Statement": [
  { "Effect": "Allow", "Action": "iot:Connect", "Resource": "arn:aws:iot:us-east-1:$ACCT:client/$CLIENT" },
  { "Effect": "Allow", "Action": "iot:Publish", "Resource": "arn:aws:iot:us-east-1:$ACCT:topic/VENKO/$KEY/*" } ] }
JSON
aws iot create-policy --policy-name venko-$KEY-gateway --policy-document file://policy.json
aws iot attach-policy --policy-name venko-$KEY-gateway --target "$(cat cert.arn)"
aws iot create-thing --thing-name $CLIENT
aws iot attach-thing-principal --thing-name $CLIENT --principal "$(cat cert.arn)"
cd ../../..

Result: five files in sites/plant5/certs/, all mode 600 — certificate.pem.crt, private.pem.key, AmazonRootCA1.pem, cert.arn, policy.json. CLIENT must equal client_id in aws.csv, and KEY the key in its topics.

## Step 6 – AWS stack (VenkoDemo)

One CloudFormation stack per site holds the storage, API, reports and dashboard. Add the site's section to VenkoDemo/infra/samconfig.toml (copy [cmd4.*], replace every cmd4, set PartitionStartDate to today, keep AllowedOrigin at * until step 7):

[plant5.deploy.parameters]
stack_name = "venko-plant5"
resolve_s3 = true
s3_prefix = "venko-plant5"
region = "us-east-1"
confirm_changeset = true
capabilities = "CAPABILITY_IAM"
parameter_overrides = "DataTopic=\"VENKO/plant5/data\" SystemTopic=\"VENKO/plant5/system\" PartitionStartDate=\"2026-10-13\" AllowedOrigin=\"*\" ReportTimezone=\"America/Mexico_City\" GlueDatabaseName=\"venko_plant5\" DataBucketName=\"venko-plant5-884520769610-us-east-1\" WebBucketName=\"venko-plant5-web-884520769610\""
image_repositories = []

[plant5.global.parameters]
region = "us-east-1"

cd ~/git_reps/claude/projects/Venko/VenkoDemo/infra
sam build --template template.yaml
sam deploy --config-env $KEY

- Bucket names are lowercase only.

- When it ends with CREATE_COMPLETE, copy the DashboardUrl output (e.g. https://d1f1ymj0tb4k52.cloudfront.net) into aws.csv's dashboard_url and rerun the build from step 3.

- Never change DataBucketName on an existing stack: the old bucket is kept and a new empty one is created.

## Step 7 – Dashboard labels, publish and CORS

The dashboard reads the sensor list from the gateway's config.json; a small overrides file adds Spanish labels, groups and decimals. Generate a starter file, edit it, then publish:

cd ~/git_reps/claude/projects/Venko/VenkoDemo
python3 tools/sync_config.py --init --source ../Akvo_Green/sites/$KEY/config_data/config.json \
    --overrides config/sites/$KEY.json

--init reuses labels from other sites where the sensor key matches and never overwrites an existing file. Example of the result after editing:

{
  "site": { "title": "Venko – Plant5", "brand": "Ghotrix - Semca" },
  "units": { "unit": "pH", "uS/cm": "µS/cm" },
  "defaults": { "warn_margin": 0.1, "decimals": 2 },
  "groups": ["Temperatura", "Ambiente", "Química"],
  "sensors": {
    "DEV_1.Pt100Sensor": { "label": "Temperatura Pt100", "group": "Temperatura", "decimals": 1 },
    "DEV_3.Humedad":     { "label": "Humedad", "group": "Ambiente" },
    "DEV_5.PhSensor":    { "label": "pH", "group": "Química" }
  }
}

Keys are DEVICE_ID.SensorName, exactly as in devices.csv (case-sensitive). Publish config and web app to the site:

bash tools/deploy_web.sh venko-$KEY ../Akvo_Green/sites/$KEY/config_data/config.json config/sites/$KEY.json

It refuses to upload if the config's topics don't match the stack's. Then lock CORS to the dashboard URL: set AllowedOrigin=\"https://<DashboardUrl host>\" in [plant5.deploy.parameters] and run:

cd infra && sam deploy --config-env $KEY && cd ..
API_ID=$(aws cloudformation describe-stack-resource --stack-name venko-$KEY \
  --logical-resource-id Api --query StackResourceDetail.PhysicalResourceId --output text)
aws apigateway create-deployment --rest-api-id "$API_ID" --stage-name prod

SAM doesn't republish the API stage when only a parameter changes, hence the create-deployment.

## Step 8 – Install on the Pi

From your computer, in Akvo_Green. The first push carries the certificates; install.sh runs once per Pi:

cd ~/git_reps/claude/projects/Venko/Akvo_Green
tools/push_site.sh $KEY pi@192.168.68.140 --certs        # add --no-modbus-tool on Raspberry Pi OS Lite
ssh -t pi@192.168.68.140 'cd Akvo_Green && ./install.sh --skip-certs && sudo systemctl start akvo-green'

- push_site.sh copies the code, the site's config.json, the certificates (with --certs), the WiFi profiles and the AKVO Modbus Tool. It asks for the Pi's sudo password for the WiFi step.

- install.sh sets up packages, the Python venv, serial-port access and the services akvo-green and akvo-history-web (and akvo-oled).

- Every later change is the same push without --certs: edit the CSVs, rebuild (step 3), tools/push_site.sh $KEY pi@<pi>. The gateway reloads config.json within 5 s.

- Push to the right Pi: pushing one site's config to another site's Pi makes that Pi publish under the wrong name and stops its own site.

Optional extras on the Pi

| Extra | How |
| --- | --- |
| Dashboard full screen at boot | ssh pi@<pi> 'cd Akvo_Green && tools/setup_kiosk.sh' (--window for a normal window, --off to remove) |
| OLED status screen | display_enabled=1 in system.csv + I2C on (step 2), rebuild, push |
| AKVO Modbus Tool | On the Pi's desktop or VNC: sudo systemctl stop akvo-green, then akvo-modbus; start akvo-green again when done |

## Step 9 – Check and record

Within about a minute of starting the gateway:

ssh pi@192.168.68.140 'journalctl -u akvo-green -n 30 --no-pager'     # "MQTT connected", no Modbus errors
aws s3 cp s3://venko-$KEY-$ACCT-us-east-1/latest/data.json - | head -c 300; echo
ssh pi@192.168.68.140 'vcgencmd get_throttled'                        # throttled=0x0 = power OK

Open the dashboard URL and confirm Gateway en línea with the dials filling in. Then write the site summary page and commit both repos (certificates and wifi.csv are git-ignored; check git status anyway):

cd ~/git_reps/claude/projects/Venko/VenkoDemo
python3 tools/site_summary.py $KEY --pi pi@192.168.68.140     # writes docs/sites/plant5.md

Checklist

- ☐ Site key chosen, lowercase, used identically everywhere

- ☐ SD card written, SSH by key, IP reserved on the router

- ☐ RS-485 port visible (/dev/ttyUSB0 or /dev/ttySC0)

- ☐ Four CSVs edited, build says Validation successful

- ☐ wifi.csv filled (if WiFi), mode 600

- ☐ Certificate ACTIVE, policy and thing attached

- ☐ Stack CREATE_COMPLETE, dashboard_url in aws.csv, rebuilt

- ☐ Overrides labelled, deploy_web.sh done, CORS locked

- ☐ Pushed with --certs, install.sh run, akvo-green active

- ☐ Data in latest/data.json, dashboard shows Gateway en línea

- ☐ site_summary.py run, site added to both READMEs, repos committed

## Removing a site

Remove in this order: the Pi first (so it stops publishing), then AWS, then the repos. The data bucket survives the stack on purpose; deleting it is a separate, deliberate step that erases the site's history. Set the variables first, as in the overview:

KEY=plant5; CLIENT=AKVO_plant5; ACCT=884520769610; PI=pi@192.168.68.140

R1 – Back up the Pi's local history (optional)

ssh $PI "tar czf ~/akvo-$KEY-backup-$(date +%F).tgz -C ~ Akvo_Green/data Akvo_Green/config_data"
scp "$PI:~/akvo-$KEY-backup-*.tgz" .

R2 – Clean the Raspberry Pi

If the Pi will become another site, skip this: push the new site over it with tools/push_site.sh <new-key> $PI --certs and restart akvo-green. To decommission it:

ssh -t $PI '
  for s in akvo-green akvo-history-web akvo-oled; do
    sudo systemctl disable --now $s 2>/dev/null
    sudo rm -f /etc/systemd/system/$s.service
  done
  sudo systemctl daemon-reload
  rm -f ~/.config/autostart/akvo-kiosk.desktop       # dashboard kiosk at boot
  sudo rm -f /etc/sudoers.d/akvo-green-reboot
  sudo apt-get remove -y akvo-modbus; rm -rf ~/.local/share/akvo-modbus
  rm -rf ~/Akvo_Green ~/.config/akvo-kiosk
'

The WiFi profiles akvo-wifi1/akvo-wifi2 are left on purpose: removing the one the Pi is using cuts your SSH session. Remove them only from a Pi on a cable, or at the site: sudo nmcli connection delete akvo-wifi1 akvo-wifi2.

R3 – Delete the AWS stack

Empty the web bucket first (CloudFormation won't delete a non-empty bucket), then delete the stack. This removes Firehose, the IoT rules, Glue/Athena, the Lambdas and API, CloudFront and the web bucket — everything except the data bucket.

aws s3 rm s3://venko-$KEY-web-$ACCT --recursive
aws cloudformation delete-stack --stack-name venko-$KEY
aws cloudformation wait stack-delete-complete --stack-name venko-$KEY

Use aws cloudformation delete-stack rather than sam delete: sam delete crashes with NoSuchBucket when a bucket is already gone.

R4 – Delete the IoT certificate, thing and policy

They aren't part of the stack (step 5 made them by hand). Run from the Akvo_Green root:

ARN=$(cat sites/$KEY/certs/cert.arn); CERT_ID=${ARN##*/}
aws iot detach-thing-principal --thing-name $CLIENT --principal "$ARN"
aws iot detach-policy --policy-name venko-$KEY-gateway --target "$ARN"
aws iot update-certificate --certificate-id $CERT_ID --new-status INACTIVE
aws iot delete-certificate --certificate-id $CERT_ID
aws iot delete-thing --thing-name $CLIENT
for v in $(aws iot list-policy-versions --policy-name venko-$KEY-gateway \
    --query 'policyVersions[?!isDefaultVersion].versionId' --output text); do
  aws iot delete-policy-version --policy-name venko-$KEY-gateway --policy-version-id $v
done
aws iot delete-policy --policy-name venko-$KEY-gateway

The loop removes older policy versions; delete-policy refuses a policy that still has them.

R5 – Delete the data bucket (deliberate: erases all history)

Only when the site's readings, reports and history are really no longer needed. Download what you want to keep first:

aws s3 sync s3://venko-$KEY-$ACCT-us-east-1 ./venko-$KEY-archive     # optional archive
aws s3 rb s3://venko-$KEY-$ACCT-us-east-1 --force

R6 – Clean the repos and commit

| Repo | Remove |
| --- | --- |
| VenkoDemo | the [plant5.*] sections in infra/samconfig.toml; config/sites/plant5.json; docs/sites/plant5.md and its row in docs/sites/README.md; the site in README's Sites in this account |
| Akvo_Green | the folder sites/plant5/ (config, certificates, wifi.csv); the site's rows in sites/README.md |

Commit and push both repos. Check that nothing points at the old site: grep -rn plant5 in both.

Removal checklist

- ☐ History backed up (if wanted)

- ☐ Pi services, kiosk, sudoers rule, Modbus Tool and ~/Akvo_Green removed — or Pi re-pushed as a new site

- ☐ Web bucket emptied, stack venko-plant5 deleted

- ☐ Certificate, thing and policy deleted

- ☐ Data bucket deleted, or kept on purpose

- ☐ samconfig, overrides, docs and sites/plant5/ removed, both repos committed

## Common problems

| Symptom | Cause | Fix |
| --- | --- | --- |
| build: FileNotFoundError: .../config_data/devices.csv | CSVs in sites/$KEY/ | Move them into sites/$KEY/config_data/ |
| sam deploy: Missing option '--stack-name' | KEY unset or no [$KEY.*] section | Set KEY, add the samconfig section |
| Stack ends in ROLLBACK_COMPLETE | Bucket or Glue database name already exists (left over, or another stack's) | Delete the old stack/bucket (aws cloudformation delete-stack --stack-name venko-$KEY), then deploy again |
| sam deploy: S3 Bucket does not exist | SAM's artifact bucket was deleted | Delete stack aws-sam-cli-managed-default, deploy again |
| Pi log: No such file … certificate.pem.crt | First push without --certs | tools/push_site.sh $KEY pi@<pi> --certs |
| Pi connects but nothing reaches AWS | Client id, topics or policy differ (often upper vs lower case) | Compare aws.csv, policy.json and samconfig.toml; all must use the same lowercase key |
| Dashboard: deploy_web.sh refuses to upload | Config topics ≠ stack topics | Fix aws.csv topics, rebuild |
| Real sensors: unexpected keyword argument 'simulate' | Known gateway bug with simulate_enabled=0 | Stay in simulate mode until it's fixed in Akvo_Green |
| akvo-modbus: no $DISPLAY | Started from SSH | Run it in the Pi's desktop/VNC, or DISPLAY=:0 akvo-modbus |
| Random reboots, SD errors | Under-voltage | vcgencmd get_throttled ≠ 0x0 → better cable or official supply |

Full reference: VenkoDemo/README.md ("Adding a site") and Akvo_Green/docs/edge_node_config/CONFIGURATION.md.